# Keysight 34461A Emulation Script Development Guide

Contents

Handy Resources	1

Outline of Emulation Script	2

Initializations and setup	2

Reset Functions	3

The Parser	4

Parameter Parsers	6

Execution Functions	6

Emulated Status Model	7

Command Table	7

The Error Queue	8

Measurement Trigger Model	9

Command Execution Functions	9

# Handy Resources

SCPI Basics:

This provides some basic information on SCPI commands and exact syntax. This is especially helpful for parsing SCPI commands.

Keysight Truevolt Series Manual:

This is the manual for all 34400 series DMM’s

DMM6500 Programmer's Manual:

This is the programmer’s manual for the DMM6500

# Outline of Emulation Script

## Initializations and setup

Lines 1-1482 (Lines based on dev version at time of writing)

At the top of the script there are two user-mutable variables. First is called gDisplayErrors which allows errors to display on the screen. The second is gErrorReadback which sends error messages back to the output queue when they are generated.

The section marked “Major structures and variables” contains many of the primary variables. Most are initialized here but populated later in the script. Note that the ‘g’ that precedes many of these names indicates “global”. You will also see “l” for local and “m” for member. These notations are left over from a previous SMU emulation script.

Aliased variables and getter/setter functions are used whenever possible in the script. While one of these may only be called once or twice in the script, the emulator is very large, and performance is key to ensure the instrument appears as an actual 34461A.

The reading buffer has its own structure. “lastRemoved” stores the timestamp of the last removed data point. On the 34461A, individual data points can be removed in order of their creation, this is not natively possible on the DMM 6500. This means that every time the buffer is accessed the start index must be checked to ensure that no “removed” data is used. Data is considered removed if its timestamp is less than “lastRemoved”.

Next are the tables that track the instrument’s state and specifications:

- gRangeTable stores the range values for all measure functions.

- gOperatingBoundaries stores the min/max/def values for many instrument attributes.

  - Commands that have a min/max/def parameter pulls from this table.

  - Notes are left in the script to indicate if each value matches the 6500 or 34461A.

- gInstrumentState stores the state of the emulated 34461A. This table is separated by which measure function the variables apply to (general if all). Each variable has the following elements:

  - value: This is the actual value of the variable, things that require a TSP enumeration to set (dmm.ON/dmm.OFF) will have their value as one of the items in "gInstrumentAttributes”

  - valueParent: A reference to the item in gInstrumentAttributes that stores the possible values for this variable.

  - getter: The corresponding getter function for this variable, necessary to programmatically check this variable

  - setter: The corresponding setter function for this variable, necessary to programmatically set/reset this variable.

The 34461A has measure function specific values for many attributes whereas the 6500 does not. (Ex. VoltDC can have one stored Rel value while resistance can have another) It is vital that the values in gInstrumentState stay synched to the 6500. Whenever possible all interactions with the 6500 should be done through this table. (ex. Set with the variable’s setter and set to the variable’s value)

- gInstrumentAttributes is a table of the possible values for state variables that require a DMM enumeration to be set. (enumerations like dmm.ON) Each attribute contains:

  - DMM_Enum

    - The enumeration for this specific attribute

  - Shorthand

    - The shorthand for this attribute that should be returned when queried

Measure function attributes have extra information:

  - stateVariables

    - A reference to the measure function’s state variables, used for systematic variable resetting

  - units

    - A string for the units of measurements

  - overload_bit

    - The bit to flip in the questionable data register when an overload reading occurs

## Reset Functions

Lines 1483-1880

The reset functions ensure the instrument and variables reset according to the specifications in the 34461A manual. These lead to opportunities for a desynch between the script and instrument state. Special care must be taken to avoid this.

CheckTerminals checks if the front or rear terminals are currently being used and updates the current sensing capabilities accordingly. (Rear can go up to 3A) This should be called whenever measure functions are changed to current or current related variables that depend on measure ranges are used to ensure the emulated state matches the instrument state.

gResetFunctionVariables is a table of functions that reset the instrument state variables to their default settings. It is important to ensure these values match the 34461A defaults as close as possible. gSetFunctionAttributes is very similar but it sets the 6500 settings to match the corresponding state variables.

## The Parser

Lines 1881-2200

The parser is the most important part of this script. The Command table is the main parser data structure. It is a tree that reflects the command structure of the simulated SCPI instrument. At each level, an element could be a non-terminal (path) element or a terminal command.  For terminal commands, there can be a command form and/or a query form.  The command might also take parameters. The following generic structures are used to hold the command tables:

Header Element:

- (mPath)

  - Header Element sub-tree (for path, when next char is a :)

  - (This member is implied. For efficiency, its members are stored directly at this level. See comment below.) mPath is the series of keys used to get to the current header.

- mCommand

  - Command element for a command (if not a path) )(See command element below)

- mQuery

  - Command element for a query (if not a path)(See command element below)

Command Element:

- mExecute Function to execute when command/query is run.

- mParameters Array of parameter elements for parameters.

Parameter Element:

- mOptional   True if parameter is optional. False or nil if not.

- mDefault    Default value. (Not used in this script)

- mParse      Parser function. (These are defined in gParserTable)

- mData

  - This is only needed when mParse is set to ParseParameterChoice. This is when a single parameter may be a number of possible types that require different parsers. mData should be a table of parser functions.

- mNames

  - This is used when mParse is set to ParseParameterName. This should be a table of name keys and their return values.

For SCPI based instruments, command path strings are not case sensitive. We can utilize this to simplify building the command table structures. Instead of creating an explicit mPath member, we can simply encode all the sub-tree elements with upper case keys and put them directly in the table of the parent command element. Observe that all member names start with a lower case "m" and will not conflict with any child element keys.

ParseInfo

This structure holds a partially parsed command. The parameters have been extracted as raw strings but have not been checked or decoded.

- mCommandNode

  - The command element from the command tree.

- MCommand

  - The mQuery or mCommand element from the command node.

- mParameters

  - Array of parameter strings (partially parsed).

- mError

  - (When there is an error) The error code detected while parsing the command, or a table of error codes

Parser State

This structure maintains the input and parser state.

- mCurrentText

  - The text of the current unparsed (or partially parsed) data.

- mCurrentTextUpper

  - An upper-case copy of mCurrentText.

- mCurrentPosition

  - The byte position of where the parser will resume.

- mNextMessage

  - The next unparsed record in the record chain.

- mLastMessage

  - The last record in the record chain.

- mNextCommand

  - A ParseInfo table for the next command to process.

- mPartialMessage

  - An accumulator for a sequence of partial messages.

Each message record has the following fields:

- mType

  - The message type

- mMessage

  - The text of the message

- mNext

  - Reference to next record if there is one

- mCount

  - The count of messages received from firmware

## Parameter Parsers

Lines 2200-3828

The parameter parser section has two main parts, the parsing functions and the reusable parameter tables.

Parameter parsing functions receive the parameter as a string in lParameter. If the parsing function requires data from the parameter table, then the entire table is passed as lParseData. The function should then check that the parameter is correct, if so, returning the parameter to be used, if the parameter is incorrect the function should return nil and the error number.

The reusable parameter tables are parameter tables that are used more than once and are stored in gParserTable to simplify the script.

## Execution Functions

Lines 3829-4109

The execution functions provide multiple functions to execute SCPI commands. There are also some general-purpose functions stored here. Some functions of note:

- Initialize34461A – used to prepare the 6500 for 34461A emulation. This is a global function and is technically user accessible.

- PriorityExecute - Peek ahead for the next command and execute it if it has priority (ABORT). This should be called within functions that may take a long time and need to look ahead for possible interrupt commands.

- ScanUpdatedValues – Iterates through state variables to and checks the 6500’s state with getter functions, if a mismatch occurs then the corresponding state variable is updated. This should be used any time the instrument could be configured outside the emulation script.

- Execute34461A – Executes a single SCPI string, this is intended to be used by the user.

- Engine 34461A – This is the main loop that constantly looks for SCPI commands in the input queue and manages high level tasks.

## Emulated Status Model

Lines 4110-4347

The 34461A status model is similar to the 6500 status model but does require custom emulation. The 34461A can be found on page 386 of the manual. The four registers are represented as tables with mCondition, mEvent, mEventEnable, and mSummary when needed. Functions to manipulate these registers are defined below. One of the most important status model functions, UpdateStatusModel, is under these. This is intended to run every cycle of the engine function.

UpdateStatusModel updates many of the bits in the status model, of note are the standard event register. The OPC and PON bits are read and updated to match the 6500. Other bits are checked and updated accordingly. This function is a good place to start for any status model related issues.

## Command Table

Lines 4348-5825

Here the command table is populated with the correct paths. Each keyword is nested under its parent. If the specified path has a command or query form they are represented with a temporarily empty table called mCommand or mQuery.

Example, to represent the following commands:

SENSE:VOLTAGE:DC:RANGE

SENSE:VOLTAGE:DC:RANGE?

SENSE:VOLTAGE:DC:RANGE:AUTO

SENSE:VOLTAGE:DC:RANGE:AUTO?

The tree would look like:

[“SENSE”] = {

[“VOLTAGE”] = {

[“DC”] = {

[“RANGE”] = {

mCommand = {},

mQuery = {},

[“AUTO”] = {

mCommand = {},

mQuery = {},

},

},

},

},

}

Under the command tree aliases are used to provide shorthand versions of commands. (SENSE and SENS are valid and equivalent). I have created a development tool to check that all commands are aliased correctly. Under the command tree a function called gPrintCMDTree will traverse the tree and write all the commands to a USB. (Make sure to uncomment the export) A Python script will then take this file and compare it to Expected.txt and print any mismatches. Expected.txt must be manually populated with all expected commands. These files can be found in the developer tools folder.

## The Error Queue

Lines 5826-6142

The emulated error queue is very simple. A large table of error stores the error numbers, messages, code, and status bit. The status bit indicates what bit in the standard event register should be activated when the specified error occurs. Currently, most errors generated by the script do not match the error that would be generated by the 34461A. This is one of the largest areas for improvement for the script. The error table is accurate to the 34461A’s errors and does not need to be updated.

## Measurement Trigger Model

Lines 6143-6250

The 34461A uses a trigger model to make all measurements. However, this trigger model is far less configurable than the Keithley trigger model. This measurement model is described on page 99 of the 34461A manual. The InitializeTM function configures a 6500 trigger model to emulate the 34461A trigger model with user specified settings. InitiateMeasurements starts a set of measurements and ensures everything is properly configured. EndTriggerModel is called when the engine function detects that the trigger model has finished.

## Command Execution Functions

Lines 6251-10337

The command execution function section is the base level of the emulation script. This is where the functions for each command are defined. This is the first place to look to both understand the script and fix any issues. There are a handful of helper functions defined at the top of this section.

A general format for a command definition looks as follows:

gCurrentRoot = gCommandTree[“HEADER1”][“HEADER2”][“HEADER3”]

gCurrentRoot.mCommand.mParameters = {parameterTable1, parameterTable2}

gCurrentRoot.mCommand.mExecute = function(lParameters)

Execution function here, lParameters is table of parameters

end

gCurrentRoot.mQuery.mParameters = {parameterTable1, parameterTable2}

gCurrentRoot.mQuery.mExecute = function(lParameters)

end